Firmware Revision Information

Product Code(s): ACR1000A-CTL-8, ACR1000A-CTL-16, ACR1000A-CTL (old controller)

Product Name(s): ServSwitch iPath (old version)

Date Modified: March 13th, 2018

# Important Notice:

- You must upgrade from 3.3 only

- Do NOT perform a factory reset prior to updating to 3.5, this will disrupt the certificates, you may however perform a factory reset after going to 3.5

- You may only upload a backup of 3.3 or 3.5 revisions, any other backup will take your iPath down

# Upgrade Instructions

1. Open iPath webpage.

2. Login as an admin level user.

3. Click the ‘Updates’ click from the DASHBOARD

4. Click the file-upload button.

5. Select the ‘aim_upgrade_3.5.42099.tgz.asc’ from the location it was downloaded to.

6. Click ‘Upload’.

7. After a few minutes the upgrade will have completed.

# Version 3.5.42099

Note:  This firmware version is intended to be used with iPath firmware version 3.3.

Bug Fixes:

-  This v3.5 release contains a fix to allow new devices to be added iPath system due to certificates expiring after the maximum date allowed in a 32-bit Unix system.

-  Fixed error emails being sent from cron jobs

-  Fixed apache log files getting out of control

-  SNMP service was not responding, so this is fixed