Firmware Revision Information

Product Code(s): ACR1020A, ACR1012A-T, ACR1002A, ACR1000A-R2

Product Name(s): ServSwitch Agility

Date Modified: May 2nd, 2017

Version 4.5.40863 (5/2/2017)

Important Notes:

-Before upgrading to version 4.5, you must be at version 4.4.

-When upgrading the iPath unit to 4.5, the Agility endpoints must also be upgraded to version 4.5 to maintain compatibility.  The upgrade system forces the upgrade of the Agility units to 4.5.

-A mixed system of iPath 4.4 and Agility endpoints at 4.5 is not supported

-From March 2018, new Agility units will be s hipped with 4.5 pre-installed

-New endpoints at version 4.5 can be downgraded to 3.7 or 4.4 only.  It is not possible to downgrade them to 4.1, 4.2, or 4.3.

-If the iPath is running 4.0, it is possible to downgrade the 4.5 endpoints to 3.7 to maintain compatibility.

Firmware for this release:

| ACR1000A-CTL-xx | 4.5.40890 |
| --- | --- |
| ACR1000A-R2 | 4.5.40863 |
| ACR1002A | 4.5.40863 |
| ACR1020A | 4.5.40863 |
| ACR1012A-T | 4.5.40863 |

New Features:

-New OSD to support the ability to disable hotkeys

-Admin can now select some text to be displayed don the Receiver OSD login screen

-Channels can be marked as sensitive in the OSD

-The channel name can be presented as an on-screen overlay

-Channels have been added to the transmitter page.  The transmitter page now shows the number of channels that are associated with an individual transmitter.  By clicking on this value, you are taken to the channels page where you can configure each channel

-The API call create_channel now returns an error if the method contains information about the 2nd video head for Transmitters without the 2nd video head

-Added a new 500mS value for the hot plug signal detect period

-Channel names can support up to 15 Chinese characters.  Previously once a name exceeded 8 Chinese characters it was not displayed

-When a backup iPath failed, it was possible for the Primary iPath OSD to be slowed down.  This now has been resolved.

-When the disk was full, atd accumulated un-executable jobs, this delayed the booting of the iPath controller

-It is now possible to setup the mail server to identify which iPath in the cluster is sending an email, a subject line can be added into the email and it is now possible to use the mail server without authentication

-A backup server was added into the cluster with an incorrect zero-config ip-address

-Temporary channels were being added to the cache of the API session which meant that iPath could connect to the wrong channel

-A backup iPath server could replicate an empty iPath database which meant it could get stuck in the initializing status until fully factory reset

-When a large number of channel changes nearing 100 were executed simultaneously, the change time slowed down to 10s of seconds

-Improved card reader support which use the USB method “Clear endpoint Halt” are now supported.  This includes the Lenovo/gem plus CT40 card reader

-When using the HPZ series of PCs, the USB was lost when the PC has been rebooted.  This has now been fixed

-Support to disable OSD interaction, required to be enabled for AXIS joystick

-A SecuGen-Hamster plus fingerprint reader is now supported.  The full flow control quirk 0x2000AAA needs to be enabled.

iPath Known Issues:

-With some versions of Chrome with the option to save passwords enabled, the OSD login message and cluster password is auto-filled.  This means that when setting up a backup server in a cluster it will not join the cluster.  Workaround is to double check that the cluster password has not been auto-filled before adding a backup server or use another browser.

-The maximum number of AD users that can be a member of an AD group is 1499.  If there are 1500 or more users in a group, then the group is returned as empty.  There is currently no work around for this.

Agility Known Issues:

-It has been observed occasionally that when the OSD menu is brought up when the channel overlay is in the middle of the screen, there is video corruption to the right hand side of  the OSD where the scroll bar would be on a low video resolution.  Workaround is to enable the OSD popup messages that report when a change of video resolution occurs.

-Channel overlay rendered in the screen center has been observed.  The new OSD channel reminder banner can be mis-positioned in the center of the screen.  This has been seen a small number of times if the popup messages have been disabled.  When the video resolution changes without the popup message box enabled, the positioning information is not sent.  Opening the OSD and reconnecting the channel brings the OSD reminder banner into its correct position in the top left of the screen.  Workaround is to enable the OSD popup messages that report when a change of video resolution occurs.

-FIELD-Sticky ALT key.  When using, hotkey switching to change channels it has been seen that the hotkeys are left in the down state.  This leads to mistyped characters on the keyboard.  When this happens, pressing the hotkey releases the stuck state and you can carry on typing normally.  Workaround is if hotkey has gotten stuck down, pressing it again will release it.

Instructions to upgrade to 4.5:

Depending on the starting point of your existing system, there are three possible upgrade paths.

1.  From 4.0 iPath

Please note that if you have ACR1000A-T and ACR1000A-R units in the system you will need to stay at iPath F/W 4.0 and 3.7 on the endpoints as these units do not support 4.5 firmware.  If you are replacing the older units with newer units, you must replace them at 3.7 first before attempting the upgrade to 4.5.  This will require the new units to be downgraded to version 3.7 first.  You will need to ensure that your iPath is at version 4.0.40623 before using the replace device feature.

If upgrading from 4.0, ensure there are enough addresses in your IP address pool to allocate to all required ports.  Note:  Teaming devices will require 2 IP addresses each.

On the Primary iPath, browse to the Home>Servers TAB and ensure “Require Authentication?” is set to No.  This prevents issues when upgrading any secondary iPath servers.

If the endpoints on the 4.0 iPath are running 3.7, you can go straight to upgrading them to 4.5 as 3.7 uses 4-bit error checking on the flash.  If upgrading from below 3.7 firmware, then you will also need to go via 4.4.40349 which changes the flash to use 4-bit error checking.

2.  From 4.1, 4.2 and 4.3 iPath

If your iPath and endpoints are below 4.4 while the iPath can be upgraded straight to 4.5, the endpoints must be upgraded via 4.4 first before going to version 4.5.

However, we recommend you upgrade the iPath to version 4.4 first then upgrade all endpoints to version 4.4 prior to going to version 4.5.

Always upgrade the iPath first to 4.5, you will be prompted to upgrade the endpoints as the minimum firmware version has been set for 4.5.  If you have multiple iPaths on the network in a backup or satellite configuration, then you must upgrade all iPaths before upgrading the endpoints.  After you have updated the Primary iPath, its firmware will mismatch with the endpoints, so you will see the backup or Satellite iPaths to report themselves as active on the network until all of the iPaths have been updated.  You will then not see the endpoints until they have been updated to match.  To update the Agility units, apply the 4.4.40349 main image only.  This main image contains the main, backup and boot images (it is necessary as the Flash chips go from 1 bit error checking).  Once the endpoints are at 4.4.40349, then apply the 4.5 main and backup images to the Agility endpoints.

More details on the upgrade process for Agility endpoints below 4.4:

- Ensure all devices are connected and online in iPath

- Upload the 4.5 iPath firmware package to the Primary iPath server.

- Once the firmware is uploaded into its partition, the secondary iPath will become acting primary.  The primary temporally stops responding to the backup as the updated is applied.  You will need to redirect your browser back to the Primary IP address.  The primary will become available once the 4.5 image has been uploaded.

- On browsing back to the Primary, you will see that the Reboot button has appeared click this to reboot the iPath into the 4.5 firmware image.

- If you have a backup or satellite server repeat the above steps to upgrade these servers before upgrading the Endpoints.

- Login and upload the v4.4 packages for your transmitters and receivers to the Primary iPath

- Login and upload the 4.5 packages for your transmitters and receivers to the Primary iPath

- Upgrade the transmitters and receivers to main 4.4 image

- Upgrade transmitters and receivers main and backup 4.l5 images

The upgrade should now be complete, check the “Servers” TAB on the primary iPath to confirm both iPaths are upgraded and synchronized.

3.  From 4.4 iPath and 4.4 Agility

If your iPath and Agility endpoints are already running 4.4 firmware, then no special instructions apply.

- Ensure all devices are connected and online in iPath

- Upload the 4.5 iPath firmware package to the Primary iPath server

- Once the firmware is uploaded into its partition, the secondary iPath will become acting primary.  The primary temporally stops responding to the backup as the update is applied.  You will need to redirect your browser back to the Primary IP address.  The primary will become available once the 4.5 image has been uploaded.

- On browsing back to the Primary, you will see that the Reboot button has appeared, click this to reboot the iPath into the 4.5firmware image.

- Now upgrade the backup server if one is present by repeating the steps above.

- Login and upload the 4.5 packages for your TX and RX to the Primary iPath

- Upgrade Transmitters and Receivers main and backup 4.5 images