Firmware revision information for:

Wizard IP Plus

Important note: If you are upgrading from version 0.3, 0.4 or 0.5 then you must apply version 1.0 upgrade before applying later upgrades.

# Version 0.3

Original shipping release.

# Version 0.4

1. File transfer changes. General changes to how the mass storage device is handled.

2. Various other minor user interface changes.

# Version 0.5

February 2007

1. New DHCP client. This version includes a new DHCP client which resolves a problem with system lock-outs when using dynamically allocated IP addresses (i.e. not fixed IP addresses).

2. Various other minor user interface changes.

# Version 1.0

March 2007

Version 1.0 contains some major structural changes to the code and so requires a two stage upgrade process. Once upgraded to version 1.0, it will not be possible to downgrade to previous code versions. The upgrade is applied as follows:

- (STAGE 1) Load the wizardipplus-upgrade-1.0 upgrade in the normal way as instructed in the Wizard IP Plus manual.

- When you reboot back into normal mode, the unit will present a web page at the unit’s IP address prompting you to provide a Maintenance Mode upgrade file (wizardipplus-mm-upgrade-1.0.bin).

- (STAGE 2) Apply the Maintenance Mode (wizardipplus-mm-upgrade-1.0) in a similar way to the way that you applied the original wizardipplus-upgrade-1.0 upgrade.

- After rebooting back into normal mode, the unit will be upgraded to version 1.0.

Changes are as follows:

1. New menu hide/show option with configurable activation hotkey.

2. Paste clipboard dialog item added to Keyboard Control menu.

3. Bug fixes to video and mouse calibration code.

4. New magic security numbers for upgrade images.

5. Removed serial output on boot up.

6. USB keyboard now supported in maintenance mode.

# Version 1.1

April 2007

1. Additional check box added on the Configure > Unit Configuration menu to set the default state of the menu bar upon connection (i.e. displayed or not). This state will not affect any currently connected viewer, only subsequent (re-)connections.

2. Added checks for viewer resizing support before resizing due to menu bar toggling.

3. If no hotkey is defined for menu bar toggle the menu bar will be shown and the default check box will be disabled.

4. Bug fix to dma copy code should prevent some rare corruption issues - removed forced full screen update on resolution changes.

# Version 1.2

April 2007

Bug associated with operation of local USB keyboard fixed.

# Version 1.3

May 2007

1. Sound now supported through sound enabled viewers.

2. Moved digital/analogue switch to Controls/Video Settings, added new Auto mode.

3. Fixed close viewer whilst calibrating bug.

4. Fixed display activity percentage miscalculation issue.

5. Fixed file system event log problem.

# Version 1.4

June 2007

1. Fixed USB composite device incompatibility.

2. Fixed some "calibration required" indicator issues.

3. Rationalised local-only auto calibration and settings loading.

# Version 1.5

June 2007

Support for combined USB devices added to local console connection.

# Version 1.6

July 2007

This upgrade is recommended for all installations as it includes a security enhancement.

1. Security blacklist system enhanced.

2. Fixed mouse/keyboard timeout under high activity problems.

# Version 1.7

September 2007

1. Improved sound support when used in conjunction with new windows viewer (4.3se3). The viewer now no longer freezes audio playback immediately on interruption, but continues to play its remaining buffer. This technique enables the system to be more tolerant to interruptions. The viewer also now includes an option in the Misc menu: "Only play sound when window active", which will stop playback from a given viewer when it is not the active window. The new viewer (4.3se3) is compatible with older firmware and vice versa.

2. Supports new \apc and \sin power commands for easier control of APC and Sinetica RS232 controllable power switches. These commands are for large system deployments. Please contact Technical Support for more details if you require to use APC or Sinetica RS232 controllable power rails in large system deployments.

3. Sound options bug fixed (see detailed explanation below).

vncviewer4.3se2.exe sound issue

Problem description:

When the VNC viewer Option defaults are saved the sound becomes unacceptably poor on subsequent viewer sessions. The problem is associated with two parameters set in the Windows registry when the defaults are saved, PlayoutCutoff and PlayoutLen, both incorrectly being set to a value of 1. The values tell the viewer at what buffer size to start playing and at what size overflow data occurs, if they are both the same or both very low the result is either no sound or corrupted sound.

Problem implications:

The values are not editable through the viewer interface so once set the values will remain in their erroneous state in subsequent viewer sessions even when the newer 4.3se3 viewer is used. Please note that the 4.3se3 viewer in itself handles saving of default values correctly, however, since it will load the erroneous defaults saved by the 4.3se2 version during start-up the problem can be carried forward.

Problem workaround:

Use the 4.3se3 viewer version with immediate effect. If version 4.3se2 has been used previously the 4.3se3 version will only work correctly for sound provided the defaults haven’t been set at any point in time. If the defaults have been set the registry on the viewing machine must be updated as follows:

(Windows PC)

•	Run regedit (take great care when running this program as a mistake may cause your operating system to become unstable)

•	In the left hand pane locate \HKEY_CURRENT_USER\Software\RealVNC\VNCViewer4

•	In the right hand pane locate PlayoutCutoff and double click to change the value to hexadecimal 7d0 (decimal 2000).

•	In the right hand pane locate PlayoutLen and double click to change the value to hexadecimal 1f4 (decimal 500).

•	Close regedit and start the 4.3se3 viewer, sound should now work correctly.

# Version 1.8

September 2007

1. Separate modem reset string added to support US Robotics 5631 and similar modems.

2. Japanese keyboard layout support added.

# Version 1.9

April 2008

Version 1.9 contains some major structural changes to the code and so requires a two stage upgrade process. Once upgraded to version 1.9, it will not be possible to downgrade to previous code versions. The upgrade is applied as follows:

- (STAGE 1) Load the wizardipplus-mm-upgrade-1.9 upgrade in the normal way as instructed in the Wizard IP Plus manual.

- When you reboot back into normal mode, the unit will present a web page at the unit’s IP address prompting you to provide a Maintenance Mode upgrade file (wizardipplus--mm-upgrade-1.9.bin).

- (STAGE 2) Apply the Maintenance Mode (wizardipplus--mm-upgrade-1.9) in a similar way to the way that you applied the original wizardipplus--upgrade-1.9 upgrade.

- After rebooting back into normal mode, the unit will be upgraded to version 1.9

Changes are as follows:

1. Fixed USB bug when used with Windows Preinstallation Environment 1.x.

2. New IPv6 support:

NTP, syslog, telnetd all support IPv6 connections.

UI (local OSD and remote) now supports IPv6 entry for relevant IP addresses.

New additional dhcpv6 client.

3. Other UI changes:

Improved local OSD text entry field functionality.

Video Settings/Custom Video Settings rationalization.

Removed extra bevel outline from remote UI.

# Version 2.0

May 2008

Version 2.0 contains some major structural changes to the code and so requires a two stage upgrade process. Once upgraded to version 2.0, it will not be possible to downgrade to previous code versions. The upgrade is applied as follows:

- (STAGE 1) Load the wizardipplus-mm-upgrade-2.0 upgrade in the normal way as instructed in the Wizard IP Plus manual.

- When you reboot back into normal mode, the unit will present a web page at the unit’s IP address prompting you to provide a Maintenance Mode upgrade file (wizardipplus-mm-upgrade-2.0.bin).

- (STAGE 2) Apply the Maintenance Mode (wizardipplus-mm-upgrade-2.0) in a similar way to the way that you applied the original wizardipplus-upgrade-2.0 upgrade.

- After rebooting back into normal mode, the unit will be upgraded to version 2.0

Changes are as follows:

1. USB gadget alterations to account for an IXP processor stepping change (ABD suffix).