Firmware Revision Information

Product Code(s): ACU5800A

Product Name(s): KVM Dual-Head Display Port CATx Extender

Date Modified: October 19th , 2016

# Upgrade Instructions:

The Wizard DP Extender transmitter and receiver both have serial Options ports which are used for firmware upgrades but are also available during normal use for transferring high speed serial data across the CATx link. This means that you can connect the serial port of the Host computer to the transmitter and any serial device to the receiver. These can then freely communicate at up to 115,200 baud across the transparent link. No serial configuration is required on the Wizard DP Extender units as they merely pass through the serial signals.

Wizard DP Extender units are flash upgradeable using the method outlined here. The same upgrade file is used to upgrade both the transmitter and receiver units (although they are upgraded individually).

Warning: During the upgrade process, ensure that power is not interrupted as this may leave the unit in an inoperable state.

1.  Download the upgrade file from the Black Box website or FTP site.
http:// ftp.blackbox.com/ connectivity/SERVSWITCH/ss_firmware/_KVM%20Extenders/ACU5800A/firmware/3.00

2.  Connect the supplied serial cable between your computer and the Options port of the Wizard DP Extender unit to be upgraded.

3. Use a narrow implement (e.g. a straightened-out paper clip) to press-and-hold the recessed reset button on the front panel until the indicators begin pulsing.

4. With the unit in download mode, transfer the upgrade file using an XMODEM file transfer via any terminal emulator program.  Use the following settings: 115200 baud, 8 bit word, no parity, 1 stop bit (8N1) and no flow control.

5. Upon completion of the download, the unit will begin to upgrade its stored firmware. Once this process has completed the unit will reboot itself and begin to operate with the new firmware.

# Firmware Revision History:

Firmware Version 3.01 – October 2017

- Mice now work on Mac OSX

- The Yen key on a 106 key Japanese keyboard is now supported

- Elo Accu-touch touchscreens are now supported don the emulated USB ports

- Known Issue:  NVidia FX3800 graphics card is not supported.  It is not possible to display any video when this graphics card is used.

- It was not possible to use Dual Touch Screens on the USB connectors if the Freedom II was used.  Workaround was to use a powered USB hub and connect both touchscreens to the USB ports or to remove the Freedom II

- Known Issue:  HP Z30W monitor.  If the computer is restarted, no video will be displayed until this monitor has been power cycled.

Firmware Version 3.00 – March 2017

- Added supported for embedded DisplayPort Audio

- Added support for 10-bit color depth / HDR.  When enabled the extender can support 1.06 billion pixels.  The graphics card and the monitor must be capable of supporting 10-bit color depth for this to be supported.

- New OSD Banner now alternates between showing the High/Low rate of operation with the mode of operation Priority/Balanced/HDR.

- Fixed key combinations.  It was possible that after pressing the CTRL the extender would enter configuration mode on the next key press.  This has been fixed so that the only way to enter configuration mode is to press and release CTRL three times.

- Fixed Caps Lock Light on the Mac Keyboard

Firmware Version 2.00 – January 2016

- Fixed issues with NUM lock / CAPS lock / Scroll Lock leds that would not flash when the hot key sequence of CTRL, CTRL, CTRL is activated.

- Support for EIZO 4K monitors.  These monitors support 4K@30Hz with reduced blanking.  Previously the extender only transferred the non-reduced blanking EDID.  Now the extender will send both the reduced blanking and non-reduced blanking EDIDs.

- It is now possible to bring the OSD menu up on a 4K monitor.  Previously it was not possible to display the OSD on a 4K monitor.

Firmware Version 1.03 – October 2014

- Minor firmware improvements to synchronise with minor hardware updates.

Firmware Version 1.02 – September 2014

- Can support 4K resolutions (3840x2160) using single stream Display Port ONLY (multi stream or MST is not supported, check your monitor specs to make sure it is compatible first).  This version has not been fully validated with monitors in the field, however we tested the monitors below:

| Monitor | Support |
| --- | --- |
| Sharp PN-K321 | Not Supported, does not support single stream |
| Dell 24” 06X55C | Supports 3840x2160 |
| Panasonic TX50-AXW804 | Supports 3840x2160 |

Firmware Version 1.0 – December 2013

- Initial Release

# USB Compatibility and Reference:

Mice Tested:

| Manufacturer | Model |
| --- | --- |
| Acer | SM-9625, M-UVACR1 |
| Asus | Mini-n6 |
| Evoluent | Vertical Mouse 3 |
| Logitech | M-U0007, MX518, G9X, G500, MX5500, Premium optical wheel mouse, Marble Mouse, M310, M505 9with unifying receiver), Wireless optical mouse (M-R006), MS570 |
| Kensington | Value optical mouse |
| Microsoft | Basic Optical mouse V2.0, Sidewinder laser gaming mouse, Optical mouse 200, wireless mouse 5000, wheel mouse optical |
| Dell | N8889, M-UVDEL1 |
| Sun | FID-638 |
| Contour | PMO-XL-R |
| Apple | A1152 |

Known Issues Mouse:

- Microsoft comfort optical mouse 3000 is not supported

- Razor Lachesis mouse is not supported

Keyboards Tested:

| Manufacturer | Model |
| --- | --- |
| Dell | KB1421, KV212-B, SK-8115, SK-8135, L100 |
| Microsoft | Wired 600 keyboard, wireless 3000 keyboard, SideWinder X6 |
| Belkin | F8E206 |
| Asus | KS2003 |
| Apple | A1243 |
| Logitech | K520, MK300, K800, Wired Illuminated keyboard, Classic 200, Y-RAY 81 |
| Cherry | ML5400 |
| Man and Machine | Simply cool keyboard 201104, Really cool keyboard MMI KR1160 |
| Acer | KU-0760 |
| Kensington | Valukeyboard PK1100U |
| Sun | Type 6, Type 7 |
| HP | KU2971, KU0316 |
| Accuratus | KYBAC260UP |
| Cyborg | V7 KU-0728 |
| Roccat | Isku ROC-12-702 |
| Ttesports | Challenger Pro |

Known Issues Keyboard:

- Dell RTD70 with build in card reader:  There is no card reader support on the “USB Port A” True Emulation ports, therefore any keyboard with a card reader will not be supported.  Plug device into “USB port B”

- Logitech G15, G19 and G110 gaming keyboards – the LCD and the Macro keys are not supported.

- Logitech MX5500 is not supported

- Razor Black Widow Ultimate is not supported