Firmware Revision Information

Product Code(s): ACX048 – ACX288, ACXC8 – ACXC080

Product Name(s): DKM FX

Date Modified: November 16th,  2016

Release (11/16/2016)
Switch:

New Features:

- Support of display port CPU extender with FZVDECPU firmware

- Support of redundant CPU boards

- Full support of DKM 576/576S port matrix

- Support of redundant controller board for DKM 48, 80, and 288 Port chassis

- Support of new extender series

- Expansion of Matrix Grid to 24 matrices

- Support of 32 user and CON macros

- Macro OSD list for executing macros

- User and CON ACL: AND/OR assignment

- Deactivation option of OSD at CON Units

- CRC32 checksum for API interface

- Multicast network communication

- Restart option for controller board

- Flash Update: New extender firmware update wizard

- Definition – CON Devices: Added OSD Disabled parameter

- System - Access: New parameter OR User/CON ACL

- System - Access: New parameter AND User/CON ACL

- System – Switch: New parameter Macro Single Step

- Support of Multicast communication

Improvements:

- Improvements of Matrix Firmware Update performance for matrix firmware >= 3.03

- Virtual CPU:  Simultaneous assignment of one real CPU to various virtual CPUs

- Accelerated update for controller boards and I/O boards

- Update process of mismatched I/O boards.

- SNMP:  Expansion of MIB with extender data for 2nd Link

- SNMP:  Traps for macros added

- Initialization process for matrix grid

- Synchronization between grid master and sub matrices

- Handling of matrix grid connections

- Handling of I/O boards

- Performance of TCP/IP interface

- USB HID communication to matrix

- Online config mode activation via configuration

- Sequential switching option for macro commands

- User login macro in Matrix Grid

- Save Status: Check device name for invalid characters

- Show configuration file version in status bar

- Matrix Grid:  When merging a configuration the new fixed port position in the grid will be calculated.  Reduced network traffic.  Improved message in message panel

- Save Status:  More detailed information while saving extender status

- Multi-Screen Control:  More detailed information in Matrix View and Grid Port View by highlighting invalid and not available CON devices

- Matrix View:  Display fiber XV I/O boards as XV instead of SFP

- Optimized memory management

- Accelerated update

Bug Fixes:

- Fixed possible blocking of Matrix View during I/O type detection

- Cat X phyter: Initialization of extenders on I/O board

- Temperature status for DKM 288 and 576

- Fix Frame option in Multi-Screen Control

- Fix port handling for USB 2.0 extenders

- Local computer switching of HDMI series

- Connection handling of previous switching

- OSD communication to extender

- User login in Matrix Grid

- XV grid line communication

- Video only switching in DKM compact matrices

- Fixed Matrix Firmware Update when using 1G and 3G I/O boards in the same matrix

- Save Status: Fixed reading detailed extender information in grid

- Matrix Grid – Merge: Fixed initialization of user

- Connect-Dialog: fixed URL encoding

- Fixed I/O port background color for unused ports

- Fixed Drag & Drop of files for JRE 1.8.0_91 on Linux and OS X

- Display ports with invalid status as invalid ports

- Definition Panels: fixed reload function

- Extender CSV Import: fixed import

CON Units:

New Features:

- Switching on Dual-Head CPUs (EXTVCON)

- <Hotkey><r><h><Enter> for HID hardware reset

- <Hotkey><r><s><Enter> for HID software reset

- New DISPLAY2 parameter selects second video channel of the dual-head units

- Deletion of Config.txt recovers factory settings

- Versioning of Config.txt file

- Various new audio codes for DP30Hz cards

- LOS timer in Config.txt (ENALOSTMR)

- Microphone preamplification in Config.txt (GAIN=X) for extenders with balanced audio transmitter module

Improvements:

- VGA signal per default (EXTICON)

- USB 2.0 embedded connection in Video Only mode

- Manual extender update from Windows 8 computers

- Differentiation of EDID files (dual-head units)

- Detection Dual-Head/Dual-Link (EXTDLCON)

- Support of 3440 x 1440 resolution

- Dithering filter ENADITHER (EXTCPU, EXTRCPU, EXTVCPU)

- Audio support in Video Only mode

- Bidirectional support of balanced audio upgrade module

- Automatic cutting of EDID extension block (EXTCPU, EXCTRCPU, EXTVCPU)

- Update improved

Fixed Bugs:

- Service Mode via backplane

- EDID transmission of redundant extenders

- USB 1.1 support of USB 2.0 embedded upgrade modules

- Digital audio parameters for redundant CPUs

- USB 1.1 support of USB 2.0 embedded upgrade modules

CPU Units:

New Features:

- <Hotkey><R><H><Enter> for hardware reset and <Hotkey><R><S><Enter> for software reset

- Support of USB-HID Ghosting and EDID management through the matrix

- Microphone preamplification in Config.txt (GAIN=X) for extenders with balanced audio transmitter module

- New DISPLAY2 parameter selects second video channel of the dual-head units

- Deletion of Config.txt recovers factory settings

- Versioning of Config.txt file

Improvements:

- Audio signal on both links available (redundant links)

- USB 2.0 embedded connection in Video-Only mode

- Compression control (EXTVCPU)

- Timing for memory control (EXTDLCPU)

- Detection Dual-Head/Dual-Link (EXTDLCPU)

- Support of 3440 x 1440 resolution

- Bidirectional support of balanced audio upgrade module

- Update improved

- Manual extender update from Windows 8 computers

- Differentiation of EDID files (dual-head units)

Fixed Bugs:

- Parameter RELEASETIME=X

- Link communication

- RAM timing (EXTICPU)

- Switching of redundant extenders

- EDID transmission of redundant extenders

- Audio transmission on HDMI (EXTHRCPU)

- Color defect in right half of the screen

- Digital audio parameters for redundant CPUs

- USB 1.1 support of USB 2.0 embedded upgrade modules

USB HID Firmware:

New Features:

- <Hotkey><R><H><Enter> for hardware reset and <Hotkey><R><S><Enter> for software reset

- Port deactivation in Multi-Screen Control (Trader Switch)

- Support of Logitech K400 keyboard

- Support of Logitech MK220 keyboard/mouse

- Support of various touch screens for KVM sharing

- Expanding of favorites in port mode to 254

- Support of Raspberry Pi

- Support of 32 macros in a matrix

- Freely configurable Multi-Screen Control (Trader Switch)

- Support of new X-key generation

Improvements:

- Multi-Screen Control with identical mice

- Support of Logitech Unifying Receiver

- Key buffer timeout

- Wake up of computers from sleep mode

- Boot process of Trader Switch

- Handling of multi interface devices

- Acceleration in Multi-Screen-Control

- Change of polling interval

- Handling of multi-interface keyboards

- Control of keyboard LEDs

- Fixed assignment of K/M to dedicated hub ports

- Support of horizontal scrolling of Apple mice in USB HID Ghosting

- Secure transmission of data packages

- Pointer handling in Red Hat and CentOS

- Handling of hubs for 3rd party USB switch

- Support of keyboards with mouse interface

- Support of EloTouch screens for shared control

Fixed Bugs:

- Switching of Trader Switch

- Operation of Multi-Screen Control

- Port arrangement in Multi-Screen Control (Trader Switch)

- Deactivation of ports (Trader Switch)

- Start of recovery console of mac computers

- Handling of absolute mouse reports

- Converter OSD shut down

- OSD control in cascaded environment

- Mouse handling as a combined device

- Trade Switch arrangement 2x4 in Multi-Screen control corrected

CONVGPU Firmware:

New Features:

- New resolutions for SDI (1080p@24/25/30Hz)

- EDID change via keyboard command (VGA vs DVI) <Hotkey><V><G> (VGA) or <Hotkey><V><D> (DVI)

Improvements:

- EDID transmission

- Scaling

Fixed Bugs:

- RAM timing

- Saving of image settings

- Minor bug fixes

Java Tool:

New Features:

- System - Matrix Grid: New Offline Grid Wizard in status and configuration files

- Assignment – Virtual CPU Assignment: Allow assignment of a real CPU to multiple virtual CPUs

- Macro – Assign CPU, Assign CON: Added Remove Assignment parameter

- Support of display port cpu extender with FZVDECPU firmware

- Support of redundant CPU boards

- Flash Update: New extender firmware update wizard

- Definition – CON Devices: Added OSD Disabled parameter

- System - Access: New parameter OR User/CON ACL

- System - Access: New parameter AND User/CON ACL

- System – Switch: New parameter Macro Single Step

- Support of Multicast communication

Improvements:

- File Upload/Download communication improved

- System – Matrix Grid: Improved communication between TeraTool and matrix in Online Grid Wizard

- Reduced communication between Java Tool and matrix

- Significant improvement of Matrix Firmware Update performance for matrix firmware >= V03.03

- Save Status: Check device name for invalid characters

- Show configuration file version in status bar

- Matrix Grid: When merging a configuration the new fixed port position in the grid will be calculated.  Reduced network traffic.  Improved messages in message panel.

- Save Status: More detailed information while saving extender status

- Multi-Screen Control: More detailed information in Matrix View and grid port view by highlighting invalid and not available CON Devices

- Matrix View: Display fiber XV I/O boards as XV instead of SFP

- Fixed Drag & Drop of files for JRE 1.8.0_91 on Linux and OS X

- Display ports with invalid status as invalid ports

- Optimized memory management

Fixed Bugs:

- Matrix View: Fixed missing SFP label for fiber cards

- Macros: Fixed incomplete macro clean up when deleting consoles or user

- System – Matrix Grid: Fixed bug in merge of macros

- Fixed possible blocking of Matrix View during I/O type detection

- Fixed Matrix Firmware Update when using 1G and 3G I/O boards in the same matrix

- Save Status: Fixed reading detailed extender information in grid

- Matrix Grid – Merge: Fixed initialization of user

- Connect-Dialog: fixed URL encoding

- Fixed I/O port background color for unused ports

- Definition Panels: fixed reload function

- Extender CSV Import: fixed import