ServSensor Camera Firmware Update

Important: This procedure applies only if the firmware of your ServSensor Camera is CP-MXLV342 or later. If you have an older version, please contact our tech support team.

- Prepare necessary files for the upgrade

Please copy the following files on a Windows PC in a separate folder:

  - cp-371d_step1.pkg (5.8 MB)

(md5sum: bf9551ed9f5f995fd8fa7c63cc464e84)

  - cp-371d.bin (35.7 MB)

(md5sum: 8539453092f8def23ac67defc457480a)

Note: you may use the included “md5sum.exe” tool to check the MD5 signatures.

- Connection

We recommend connecting ServSensor Camera to a Windows PC used for the upgrade process via a “cross-over” cable.

Please make sure there will be no power outages during the upgrade process and you have a reliable connection between the PC and ServSensor Camera during the whole update process, otherwise the update may fail and the unit may become unusable.

- Apply the latest patch

- Open the web interface of the ServSensor Camera; click the "System" tab.

- At the bottom of the page, click the "Upgrade" button.  The pop-up window saying "This will reboot the device into update mode. Do you want to reboot?" is displayed.  Click "Yes" to reboot the ServSensor Camera.  The ServSensor Camera will then automatically reboot into a safe mode.  The web interface will be redirected to the safe mode page in about 30 seconds.

- Next, click the "Browse" button and navigate to the directory holding the file “cp-371d_step1.pkg”.  Select this file to open.

- Click the "Upgrade" button.  The process will take about 30 minutes.

- The ServSensor Camera will automatically reboot to the safe mode again.  You will continue with the next file.

- Click the "Browse" button and navigate to the directory holding the file “cp-371d.bin”.  Select this file to open.

- Next, click the "Upgrade" button.  The process will take about 60 minutes.

The upgrade should finish without any error, and it should say "Upgrade Complete".  The ServSensor Camera will automatically reboot to the normal mode.

Note: You may receive a timeout error message from the web browser while it is reloading a web interface.  Just ignore it, and open the web interface by entering the IP address of the ServSensor Camera to the web browser.

The upgrade is now finished.  On the web interface, click the “System” tab.  On the System Description line, it should say the version number of “CP-MXLV371d”.