Model  IC830A & IC831A

USB to RS-485 Mini-Converter

Features

- Connects an RS-485 2-Wire or 4-Wire device to your USB Port

- USB Port Powered

- USB 2.0 Compatible

- RS-485 Data Rates up to 921.6 kbps

- Automatic Configuration on Windows 2000, XP, Vista & Windows 7

Functional Description

Universal Serial Bus (USB) has become the connectivity workhorse of today’s PCs, replacing the familiar serial ports. However, many commercial and industrial devices still use the RS-485 interface. To connect these devices to modern PCs, you need a simple and reliable conversion solution. The IC830A and IC831A offer this solution in a space saving, USB Port powered package. Simply plug the converter into an available USB port on your computer or USB hub and install the drivers supplied on CD ROM. The device will show up as an additional COM port in the Windows Device Manager which is fully compatible with your Windows applications. A one-meter USB cable is included

Operation

- Drivers are included on the Compact Disk included with the converter. Simply connect the converter to an available USB port and insert the CD into the drive. The “Found New Hardware Wizard” will guide you through the installation process. The drivers are not available via Microsoft Windows Updates. When prompted to connect to Windows Updates to search for drivers, select “No, not at this time” and follow the instructions for installing from the CD.

- When the driver software is installed, the converter will show up in Windows Device Manager as the next available COM port labeled “RS-485 Port.”  Configure the COM port for your application. The default setup is 9600 baud, 8 data bits, no parity, 1 stop bit, no flow control. The converter will also be listed under USB Controllers.

- To uninstall the drivers, follow the instructions contained in the uninstall “read me” file.

- Use the IC830A for RS-485 2-Wire systems. In the 2-Wire configuration, the receiver toggles with the transmit data. Bias is provided by 4.7 KΩ pull-up and pull-down resistors.

- Use the IC831A for RS-485 4-Wire systems. In the 4-Wire configuration, the receiver is always active. Bias is provided by 4.7 KΩ pull-up and pull-down resistors on the receive pair. Bias should be provided on the transmit pair by the end of line slave device.

- In most applications, termination is not required. If your application does require termination, it must be installed external to the converter.

Ordering Information

| Model Number | Description |
| --- | --- |
| IC830A | USB to RS-485 2-Wire Converter |
| IC831A | USB to RS-485 4-Wire Converter |

| Table 1 |  |  |  |  |  |  | General Specifications |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DB9F Pin-out |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PIN | IC830A | IC831A |  |  |  | Input Power |  |  |  | 5VDC from USB Port |  |  |  |  |  |  |  |
| 1 | Not Used | Not Used |  |  |  | Current Draw |  |  |  | Lower Power Device (<100 mA) |  |  |  |  |  |  |  |
| 2 | Data - | RDA (-) |  |  |  | Operating Temperature |  |  |  | -40 to 176 °F (-40 to 80 °C) |  |  |  |  |  |  |  |
| 3 | Data + | TDB (+) |  |  |  | Operating Humidity |  |  |  | 0 to 95% Non-condensing |  |  |  |  |  |  |  |
| 4 | Ground | Ground |  |  |  | USB Connector |  |  |  | USB type B Female |  |  |  |  |  |  |  |
| 5 | Not Used | Not Used |  |  |  | RS-485 Connector |  |  |  | DB9 Female |  |  |  |  |  |  |  |
| 6 | Ground | Ground |  |  |  | USB Data Rate |  |  |  | 12 Mbps |  |  |  |  |  |  |  |
| 7 | Data + | RDB(+) |  |  |  | RS-485 Data Rate |  |  |  | 921.6 kbps |  |  |  |  |  |  |  |
| 8 | Data - | TDA (-) |  |  |  | USB Standard |  |  |  | 2.0 (Backward Compatible) |  |  |  |  |  |  |  |
| 9 | Not Used | Not Used |  |  |  | Dimensions |  |  |  | 2.3 x 1.3 x .6 in (5.8 x 3.2 x 1.6 cm) |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MTBF (IC830A) |  |  |  | 2130833 hours |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MTBF (IC831A) |  |  |  | 1869313 hours |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Approvals |  |  |  | CE, FCC |  |  |  |  |  |  |  |
| CE Declaration of Conformity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Model Number: |  |  | IC830A and IC831A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description: |  |  | USB to RS-485 Converter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type: |  |  | Light Industrial ITE Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Application of Council Directive: |  |  | 89/336/EEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standards: |  |  | EN55022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | EN 61000-6-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | EN 61000 (-4-2, -4-3, -4-4, -4-5, -4-6, -4-8, -4-11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |