General

- After inserting the CD, the  USB Driver Installation wizard should launch automatically.  If not, navigate to the CD-ROM drive and run setup.exe.  To find your CD-ROM drive, double click “Computer” or “My Computer” on your desktop, then double click on your CD-ROM drive (usually D:).

- In Windows Vista, if the following dialog appears, click “Run setup.exe”.

- When the USB Driver Installation Wizard appears, follow the wizard to complete the installation.

- When the USB Driver Installation Wizard is done installing the driver, using the included USB cable plug the USB hardware into an available USB port on the PC.

1.	One USB port is required for each installed device.  The USB port can be native to the PC or it can be a USB port from an installed USB hub to the PC.

Note: The devices work with USB 1.1 or 2.0 ports but have a maximum USB data rate of 12Mbps.

Surrounding Air Ambient Temperature: 0 to 70° C

Items Included

Installation

Important!

Install the CD Drivers before Connecting the Hardware to the PC.

Quick Start Guide

USB to 1 Port Serial and

1 & 2 Port DIN Converters

- USB to Serial Device

- One Meter USB Cable

- CD ROM with Drivers

- This Quick Start Guide

- INSERT THE DRIVER CD INTO YOUR CD-ROM BEFORE PLUGGING THE HARDWARE INTO THE PC.

- If you have inserted the hardware first and the Found New Hardware Wizard launched, click cancel to exit the wizard.

- USB to Serial Device

- One Meter USB Cable

- CD ROM with Drivers

- This Quick Start Guide

Dip Switch Setting

To verify the installation went correctly open the Windows Device Manager

Scroll down to Ports,

Expand the ports by clicking on the plus sign (+), this shows if the ports now exist on the PC.

If there are no exclamation points or other indicators of a problem the ports should be installed correctly and ready for use.

Verifying with a loopback test.

If the device is RS-232, loopback pins 2 and 3.  If the device is RS-422 or RS-485, loopback the TDA(-) to RDA(-) and TDB(+) to RDB(+).  Use the pin-out charts for the location of each pin or terminal.

Using Hyper Terminal or similar program, connect to the appropriate COM port. Set the desired baud rate. Ensure Hyper Terminal local echo is OFF.  (Note: Hyper Terminal is not provided with Vista or 2003 Server)

Transmit data. If the same character string is returned, the test is good.

Verifying Installation

DB9 Male

RS-232

NOTE:  To remove drivers from a PC, there is an Uninstall reference document on the CD ROM.

DB9 Female

Note:  For models with selectable RS-422/485 configurations

Dip switches allow the module to be configured for two-wire or four-wire, RS-422 or RS-485 modes. In two-wire mode the TDA (-) and RDA (-) are tied together and so are TDB (+) and RDB (+), making multi-dropping this converter into an existing network easy.

Pinouts