IC834A

USB to RS-232 Mini-Converter

- Connect RS-232 Devices to your USB Port

- Perfect for Field Service Applications

- Small – Fits easily into any laptop bag

- USB Port Powered

- USB 2.0 (12 Mbps) Compatible

- RS-232 Data rates up to 460.8 Kbps

- Supports Windows 2000, XP, Vista & Windows 7

| Specifications |  |
| --- | --- |
| Serial Technology |  |
| RS-232 | TD, RD, DCD, DTR, GND, SDR, RTS, CTS, |
|  | RI |
| Connector | DB9 Male (DTE) |
| Data Rate | Up to 460.8 Kbps |
| USB Technology |  |
| Connector | USB Type B Female |
| Standard | 2.0 (Backward Compatible) |
| Data Rate | 12 Mbps |
| Power |  |
| Source | USB Port |
| Input Voltage | 5 VDC |
| Consumption | ~ 0.5 W (Low power device, draws less than |
|  | 100 mA) |
| Software |  |
| Driver CD | Windows 2000, XP, Vista & Windows 7 |
| Mechanical |  |
| Dimensions | 2.3 x 1.3 x .6 in (5.8 x 3.2 x 1.6 cm) |
| Enclosure | In-line mounted, plastic |
| Weight | 0.23 lbs (104.3 g) with USB Cable |
| Environmental |  |
| Operating Temp | 32 to 158°F (0 to70 °C) |
| Storage Temp | -40 to 185 °F (-40 to 85 °C) |
| Op Humidity | 0 to 95 % (Non-condensing) |
| MTBF | 1946086 hours |
| MTBF Method | Parts Count Reliability Prediction |
| Regulatory |  |
| Approvals | FCC, CE |
| Ordering Information |  |
| Model Number | Description |
| IC834A | USB to RS-232 Mini-Converter |

Universal Serial Bus (USB) has become the connectivity workhorse of today’s PCs, replacing the familiar serial ports. However, many commercial and industrial devices still use the RS-232 interface. To connect these devices to modern PCs, you need a simple and reliable conversion solution. The IC834A offers this solution in a space saving, USB Port powered package. Simply plug the converter into an available USB port on your computer or USB hub and install the drivers supplied on CD ROM. The device will show up as an additional COM port in the Windows Device Manager which is fully compatible with your Windows applications. A one meter USB cable is included.

Package Contents

Loop Back Test

- IC834A USB  to RS-232 Mini-Converter

- 3ft (1M) USB Cable

- Datasheet

- CD with Drivers for Windows 2000, XP, Vista & Windows 7

- Loop back pins 2 and 3 on the DB9 M connector.

- Using Hyper Terminal or similar program, connect to the appropriate COM port. Set the desired baud rate. Ensure Hyper Terminal local echo is OFF.

- Transmit data. If the same character string is returned, the test is good.

DB9 Pin-out

| RS-232 Pin-out DB9 Male DTE |  |  |
| --- | --- | --- |
| Pin | Direction | Signal Name |
| 1 | Input | DCD (Receive Line Signal Detector) |
| 2 | Input | RD (Receive Data) |
| 3 | Output | TD (Transmit Data) |
| 4 | Output | DTR (DTE Ready) |
| 5 | N/A | SG (Signal Ground) |
| 6 | Input | DSR (DCE Ready) |
| 7 | Output | RTS (Request to Send) |
| 8 | Input | CTS (Clear to Send) |
| 9 | Input | RI (Ring Indicator) |