Firmware Revision Information

Product Code(s): KV0081A, KV0161A

Product Name(s): ServSwitch CX Uno

Date Modified: January 2nd, 2019

Version 1.08

- On power cycle the mouse is sometimes non-functional

- Problem with HID report parsing

- The CTRL or ALT key held down

Known Issues

- Man and Machine Really O’Cool keyboard doesn’t work on the local console, but you can use it on the remote user station just fine

- It is not possible to launch the OSD using the hotkeys + M on a non UK keyboard.  In order to launch the OSD you need to use a character where the M would be on a UK keyboard, the “,” on a French keyboard

Version 1.07b

- Updated Dual Access SAMs firmware to version 1.02

Version 1.07a

- Updated USB SAM firmware to version 2.00

- Updated Dual Access SAMs firmware to version 1.01

Version 1.07

- Added support for two page EDID

- Major changes to USB host code.  Now supports multiple keyboards and mice

- Added an RC8 compatibility mode option

Version 1.06

- Added support for the KVM Trays (created KVM modules)

Version 1.05

- Added Sun key re-map

- Included setup option to allow the OSD to be launched from a local console on a cascaded unit

- Updated the USB host code to improve reliability during enumeration

Version 1.04

- Release firmware