KV171x User Stations

Version 3.1.0.1

Release Notes

March 23, 2005

This document outlines:

1. KV171x User Station Firmware Version

2. KV171x User Station Compatibility

3. How to upgrade

4. Enhancements/Fixes/Known Issues

KV171x User Station Firmware Version Information

Firmware Information: KV171x Rev 3.1.0.1

This release is intended to be released and used with the following ServSwitch Octopus

firmware/software revisions:

- Octoware software     revision 2.1.0.9

- KV170x switch           revision 3.3.0.19

- SAM-232                    revision 1.0.1.0

- SAM (all other types)  revision 4.0.8.0

KV171x User Station Compatibility

This release is compatible with the following:

Octoware software               From revision 2.0.0.0

KV1701 switch                    From revision 3.0.0.1

KV1702 switch                    From revision 3.0.0.1

KV1720A    SAM-PS2         From revision 4.0.2.0

KV1721A    SAM-USB        From revision 4.0.2.0

KV1722A    SAM-MD8       From revision 4.0.2.0

KV1724A    SAM-232         All revisions

This release introduces new features that will only operate with the following:

Octoware software              From revision 2.1.0.9

KV170x switch                    From revision 3.3.0.19

KV1720A      SAM-PS2       From revision 4.0.8.0

KV1721A      SAM-USB      From revision 4.0.8.0

KV1722A      SAM-MD8     From revision 4.0.8.0

KV1724A      SAM-232        All revisions

How to upgrade the KV171x user stations

You may update the firmware from Octoware software revision 2.1.0.9 or from your

current revision of Octoware software.

We recommend that you first update Octoware software to revision 2.1.0.9.

We also recommend that you update the KV170x matrix switches to 3.3.0.19 before

updating user stations.

1. From your Octoware software, Select 'Admin Tools - Firmware Update' from the

menu.

2. Select all of the KV171x user stations on your configuration.

3. From the "firmware update" dialog, select the update file for the KV170x

switch devices. This file will be named "kv171p2.bbx" for KV1711/KV1712

Devices and "kv171x.bbx" for KV1710 Devices. Ensure you select the revision

3.1.0.1.

4. Select "begin update", and all firmware updates will complete. You may update

all of your KV171x user station devices in parallel for fastest results.

ENHANCEMENTS

This release of KV171x user station firmware contains the following

enhancements:

1. All printable-ASCII characters are now allowed to be entered in the password

field at the login prompt:

This is to facilitate the Remote User Authenticaton (i.e. LDAP) system

feature.

Note: The Remote User Authentication feature is not configurable from the

OSD of the KV171x user stations. This feature is supported by the

KV171x user stations, but it must be enabled and configured using

Octoware software version 2.1.0.9.

2. Support for Pinnacle Post Deko FAK Keyboard (only available on KV1712 user

station)

3. Support for standard PS/2 keyboards has been enhanced.

4. Usability Features have been added:

- Search by UID

- Server Hotkey Assignment

- User station lockdown

- User selection of Target Flag settings

- Force Mode from Command Line

- Private Mode Lockdown

- Enhanced OSD Layout

5. Seamless Features; Seamless switching interface is now supported from the

KV171x OSD through other KVM switches:

List of supported KVM switches

ServSwitch IP KV120

ServSwitch IP KV121

ServSelect II KV2016

ServSwitch Multi

Generic KVM Switch

FIXES

This release of KV171x firmware contains the following fixes:

1. Critical-error recovery has been added. Previously, if the user station

could not successfully detect an attached KV170x matrix switch, it could

get stuck in a state displaying "Searching for Matrix or Target".

This is no longer possible. The user station will timout from this state,

and will present the OSD.

2. Enhanced Bootloader functionality

3. Faster Firmware-Update ability

KNOWN ISSUES

None