Octoware® Software

Version  2.1.0.9

Release Notes

March 21, 2005

This document outlines:

1. Octoware Software Version Information

2. Octoware Software Compatibility

3. Important installation guidelines

4. How to upgrade

5. Enhancements/Fixes/Known Issues

Octoware Software Version Information

Version Information: Octoware 2.1.0.9

This version of Octoware software is intended to be released and used with the

following ServSwitch Octopus firmware revisions:

KV170x                     revision 3.3.0.19

KV171x                     revision 3.1.0.1

SAM-232                    revision 1.0.1.0

SAM (all other types) revision 4.0.8.0

Octoware Software Compatibility

This release is compatible with the following firmware:

KV170x     switch                From revision 3.0.0.1

KV1710     user station        From revision 1.2.2.38

KV1711     user station        From revision 1.2.2.39

KV1712     user station        From revision 1.2.2.39

KV1720A	SAM-PS2           All revisions

KV1721A	SAM-USB          All revisions

KV1722A  SAM-MD8         All revisions

KV1724A   SAM-232           All revisions

This release will successfully update firmware from all of the following

revisions:

KV170x     switch                All revisions

KV1710     user station        All revisions

KV1711     user station        All revisions

KV1712     user station        All revisions

KV1720A	SAM-PS2           All revisions

KV1721A	SAM-USB          All revisions

KV1722A  SAM-MD8         All revisions

KV1724A   SAM-232          All revisions

This release will introduce new features that will only operate with the

following revisions:

KV1701   switch             From revision 3.3.0.19

KV1702   switch             From revision 3.3.0.19

KV1710   user station     From revision 3.1.0.1

KV1711   user station     From revision 3.1.0.1

KV1712   user station     From revision 3.1.0.1

KV1720A  SAMPS2       From revision 4.0.8.0

KV1721A  SAM-USB     From revision 4.0.8.0

KV1722A  SAM-MD8    From revision 4.0.8.0

KV1724A  SAM-232      All revisions

Important Installation Guidelines

You should be aware that when you update to this revision of Octoware software,

the database tables will be modified and updated to a new format that includes

extra feature information about the system.

This means that pre-2.1.0.9 revisions of Octoware software may not be able to

either open or read the newer database files.

i.e.

If you update Octoware software, you cannot subsequently downgrade to a

previous revision of Octoware software without first un-installing and

re-installing. The process of un-installing and re-installing Octoware software

will cause you to lose your database files.

Therefore, if you think that you may wish to downgrade back to a pre-2.1.0.9

revision of Octoware software sometime in the future, you should make a backup

copy of the database files. The database files that you would need to backup

are: msa.data, msa.properties, msa.script and msa.backup

These database files will be located in the Octoware software installation

folder, which is defaulted to : c:\program files\blackbox octoware\

When making a copy of the database files, it is important that Octoware software

is not running. Likewise, when replacing the database files, it is also

important that Octoware software is not running.

How to upgrade

When you update to Octoware software 2.1.0.9, it is also recommended that you

update the firmware on your kv170x devices to revision 3.3.0.19 in order to

enable some of the extra features Octoware software offers to the ServSwitch Octopus

switching system.

The kv170x switch devices must be at revision 3.3.0.19 to allow the extra

ServSwitch Octopus system features to take effect.

The kv171x User station devices must be at revision 3.1.0.1 to allow the extra

ServSwitch Octopus system features to take effect.

The SAM devices must be at revision 4.0.8.0 to allow the extra ServSwitch Octopus system

features to take effect.

You may update your firmware either before or after you update Octoware

software.

Completing the installation

1. Run the Octoware software. You will be prompted for a password. The default

password is 'password'. We recommend that you change the password from its

default setting.

2. After updating Octoware software, set the Network Settings from the Admin

menu.

3. You now need to either “Discover” or “Add” your kv170x switch devices.

Note: If Octoware software exists on the same physical network and subnet as

the kv1701 or kv1702 switches, then you will only need to perform a “Discovery”

at this step, and all of your kv1701 and kv1702 switches will be automatically

discovered. If they are not automatically discovered, check that the ConfigID

specified in you Network Settings matches the Config ID specified on all of

your kv1701 or kv1702 switches.

Note: If you have not already updated the firmware on your kv1701 or kv1702

switches, then you may not be able to successfully synchronize the system

databases. You will need to update the firmware on the kv1701 or kv1702

switches, then continue to step 4.  For information on upgrading the kv1701

or kv1702 switches, see the support section for these switches.

4. When you have either "discovered" or "added" each of the kv1701 or kv1702

switches, perform a “Synchronize System Database” on your newly configured

system.

During installation, if you chose to 'delete' the old Octoware database,

then:

4.a. Choose one of the kv1701 or kv1702 switches as your Database Source.

4.b. Use the ‘overwrite’ option.

4.c. Click the start button.

Otherwise (you did not delete the old Octoware database):

4.d. Choose Octoware as the database source.

4.e. Use the 'overwrite' option.

4.f. Click the start button.

Enhancements

This release of Octoware software contains the following enhancements:

1. Ability to set up Remote User Authentication using LDAP (Lightweight

Directory Access Protocol) on a Blackbox switching system.

2. Support for nVision Advanced Video and Audio Switching in tandem with

ServSwitch Octopus  video switching.

3. User has the ability to set individual OSD preferences (User Settings Tab).

4. Ability to view/setup non-Octopus KVM switches cascaded through an

Octopus-SAM module:

List of supported KVM switches

ServSwitch IP     KV120

ServSwitch IP     KV121

ServSelect II       KV2016

ServSwitch Multi

Generic KVM Switch

Fixes

This release of Octoware software contains the following fixes:

1. Added button group to radio buttons in log event dialogs to prevent

multiple selections.

2. When logging out a user from a KV171x user station from the 'Logged in

Users' window, an error message was displayed even though a KV171x user

station was selected

3. Renaming a user station could have caused "unable to connect to database:

null" error.

4. Removing offline servers/switches caused the first entry to be removed.

5. Lost 'subnet' information on the Device Discovery panel.

6. Synchronizing using Octoware software on a multi-processor platform running

Windows 2003 Server.

7. Context sensitive help on the Devices->Edit/View Matrix Switch sub-screen

is available in this release.

8. As with previous versions of Octoware, when performing a

database-synchronization in overwrite mode (taking an Matrix switch as

the database source), all grouping information will be lost on Octoware.

In order to ensure that grouping will not be lost inadvertently, we have

introduced a warning message when this option is selected.

9. As with previous versions of Octoware, we recommend that when using Groups,

Administration should be done only from Octoware. However, we have made the

following change in order to maintain system synchronization in case of

inadvertent administration via the user station:

When a user is associated with one or more groups, and subsequently that

users' rights are modified from a User Station, then that user will be

automatically removed from groups on Octoware, and Octoware will remain in

sync, with the users individual rights reflecting the user’s effective

rights prior to removal from group.

10. End User License Agreement updated.

NOTE:

When Octoware software is running on Windows XP Service pack 2:

Windows XP SP2 installs a firewall (and turns it on) by default.

This firewall blocks FTP transfer, which the Octopus switch system requires

In order to successfully perform firmware updates.

Secure Client (VPN) software may also block FTP transfer.

In order to successfully update firmware, you may need to switch off your

firewall (and/or Secure Client software) for the duration of the firmware

updates.

Known Issues

This release of firmware/software contains the following known issues for

Octoware software:

1. Not Applicable