KV1720A Module

PS/2 Server Access Module

Firmware Version 4.0.8.0

Release Notes

March 21, 2005

This document outlines:

1. KV1720A Module Firmware Version

2. KV1720A Module Firmware Compatibility

3. How to upgrade

4. Enhancements/Fixes/Known Issues

5. Fixes

6. Known Issues

1. KV1720A Module Firmware Version

Version Information:

Application Code  4.0.8.0

Boot Code            4.0.6.0

FPGA                   2.0.8.0

This version of  the KV1720A firmware uses samps2.bbx files for upgrade.

2. KV1720A Module Firmware Compatibility

This revision is compatible with the following firmware:

KV170x switch:

- Basic functionality with all revisions.

- Full feature support requires 3.3.0.19 or higher.

KV1710 user station:

- Basic functionality with all revisions.

- Full Japanese and Korean layout support with version 1.1.6.3 or higher

only.

- No Extended Keyboard Support.

- Full feature support requires 3.1.0.1 or higher.

KV1711 user station:

- Basic functionality with all revisions.

- No Extended Keyboard Support.

- Full feature support requires 3.1.0.1 or higher.

KV1712 user station:

- Basic functionality with all revisions.

- Extended Keyboard Support with revision 1.3.0.0 or higher only.

- Full feature support requires 3.1.0.1 or higher.

Octoware software:

- Version 2.1.0.9 or higher

KV1720A:

- Version 4.0.2.0 (app) 4.0.0.0 (boot) and higher.

This KV1720A (SAM-PS2) release is compatible with the following hardware:

KV1720A:

- All hardware revisions of KV1720A.

Recommendations:

- We recommend that the following revisions of system components be used

with this revision of SAM:

Octoware software      2.1.0.9

KV171x user stations   3.1.0.1

KV170x matrix switches 3.3.0.19

3. How to Upgrade

To upgrade the flash in the SAM you will need to copy the file samps2.bbx to

a machine running Octoware software. Then follow the instructions in Octoware software for downloading the new software.

1. Download the file "samps2.bbx" to a drive that is accessible by the

machine running Octoware software.

2. Using the Octoware software, download the file to all the SAM modules that

are to be upgraded. This is done by selecting Admin Tools - Firmware Updates,

selecting the file "samps2.bbx" and the names of the SAM modules to be

upgraded, and then click begin upgrade.

4. Enhancements

This release of firmware contains the following enhancements for the KV1720A

server interface module:

1. Mouse response time improved

2. Custom selection of DDC EDID tables implemented.

3. Changed internal communication to be compatible with improved protocol.

4. Support for Pinnacle Deko FastAction Keyboard

3. Support added for Seamless KVM Switching Features;

List of supported KVM switches

==============================

ServSwitch IP KV120

ServSwitch IP KV121

ServSelect II KV2016

ServSwitch Multi

Generic (configurable) KVM switch

5. Fixes

This release of firmware contains the following fixes for the KV1720A  module:

1. DDC stop condition problem fixed

6. Known Issues

This release of firmware contains the following known issues for the KV1720A

module:

1.  Not Applicable