Firmware Revision Information

Product Code(s): KV2004A, KV2204A, KV2304A, KV2404A

Product Name(s): Wizard Desktop Switch, DVI and USB

Date Modified: April 25th, 2018

Scope

The Wizard KV family of products is designed to enable four computers to be controlled from a single KVM console.  The switch makes use of True Emulation technology which enables USB HID devices to be continuously emulated to each computer.  This unique technology enables instantaneous and reliable hotkey switching whilst also supporting the extra keys and features of enhanced keyboards and mice.

This v2.09 maintenance release contains changes to support new user configuration options to allow disabling of computer cycling by hotkey.  In addition, a fix has been made for support of the Apple A1243 keyboard in configuration mode.

Customer upgrade instructions

- Connect a Serial Cable between the PC serial port and the Wizard unit.  Turn off the unit.  Power on the unit whilst holding COMPUTER and MODE buttons.  The LED display shows a U to indicate Upgrade Mode.

- Open the KVMUploader.exe.  (Click Advanced and make sure the options cables COM port is selected).  Click ‘Query’.  The interface should show the previous version of firmware.

- Click Browse.  Select the DVIDL_209.txt file, click Open.  Click Upload Now.  Confirm (confirm again if at the same version).  The Progress bar at the bottom of the tool tracks the upload progress, which is also communicated in the panel above.

- If you want to confirm the unit has been upgraded to the correct version, plug a keyboard into the switch, and press and hold the COMPUTER button on the front of the Wizard unit, then press the “F” then “1”, then “ENTER”, and the front LED panel will show the current version.

Release April 25th, 2018

New Features:

- A new configuration option to allow disabling of the computer cycling by hotkey

- Olimex JTAG programmer supported (manufacturing plant)

- Added support for Apple A1243

Known Issues:

- HP Z440 after Reboot the USB keyboard is not present