Firmware revision information for:

ServShare II

# Version 1.50

August 2004

Initial shipping release.

# Version 1.51

September 2004

Keyboard data timeout value (when computer is busy) is increased. This modification has been made for improved RS/6000 computer compatibility when using keyboard mode 3.

# Version 1.52

November 2004

The keyboard contention strategy has been changed to resolve an obscure bug that was seen when the keyboards are working in mode 3. This bug was seen on a system where the ServShare II was connected to RS/6000 computers but the same bug may also be seen when used with Alpha and SGI computers or certain styles of KVM switch that force mode 3 keyboard operation.

# Version 1.53

November 2004

When connected to certain Wizard series extenders, the active port could briefly change to an unused extender after a timeout event. This problem was traced to an obscure bug in the way that the “video only” message (used to signal to ServSwitch Uno/Duo/Quadro products that the shared device is in use by another KVM console) was handled. This bug is fixed by this update.

# Version 1.54

August 2005

Modification added to resolve unreliable bootup of mouse link when connected to KV1515A computer interface modules.

# Version 1.55

April 2006

This version incorporates a 'compatibity engine' that works around an incompatibility issue with Avocent ® AMIQ-PS2 computer interface modules. Without this additional engine,  links between the AMIQ-PS2 module and the ServShare II lose mouse data every 5 to 10 minutes causing the mouse to get out of synchronisation and operate in an uncontrollable manner. The new engine resolves the incompatibility.