# Instructions for Upgrading ServSwich Duo/Quadro Firmware

1) Copy all files into a single directory.

2) Power down unit.

3) Connect a serial lead from Com1 of the PC to the unit.

4) Switch the upload dip switch on the rear to ON.

5) Power on unit.

6) Run one of the following programs depending the unit type –

4 port Duo – run BB2x4

8 port Duo – run BB2x8

12 port Duo – run BB2x12

16 port Duo – run BB2x16

16 port Quadro – run BB4x16

7) Select Com1 and follow instructions on the screen.

8) When the upgrade has finished power off the unit.

9) Change the upload switch to OFF and power on the unit