Firmware Revision Information

Product Code(s): KVXHP-200

Product Name(s): KVM Extender

Date Modified: February 28th 2022

Update Instructions

- Download and place the “upgrade.bin” in the root directory of a FAT32 drive.

- Connect the TX with the RX and apply power to both units

- Press and hold the Reset button on the RX until the Status LED flashes blue/red.

- Release Reset button and wait for link to re-establish the video, keyboard / mouse.

- Insert the USB flash drive into the left most USB port on the front panel of the RX.  The right one does not work.

- If you press the L-Ctrl 3 times, followed by 1, it will show the OSD and progress of the update.  It should take 8 minutes to complete.

Version 1.05 (2/28/2022)

Fixes:

- Video occasionally blanks out

- Displays sometimes lost upon server reboot

- Mouse scroll wheel events are not passed to host PC

- Support for Barco FS40-4K Projector

- Detected video resolution with Mac mini 2018 is too high, resulting in small picture.

- Video corruption issue with Mac Mini 2018

- Transparent USB doesn’t recover when the fiber or copper link is removed

Known Issues:

- Small amount of analog audio buzz has been detected when no sound is being transmitted

- About 1 in 10 upgrades appear to fail, if this happens just repeat the upgrade

- Restarting a Mac mini with OS10.14, the USB doesn’t always resume, reboot the TX.

- One Mac 4K monitor occasionally fails to resume with W201 missing video error, user has to reboot PC tor TX to get video to resume on both heads

- Dell P2418HT Touch screen functionality is intermittent, power cycle touch screen to resolve.

Version 1.04.1 (12/16/2021)

Fixes:

- Flashing red LED on power is fixed

- SiLabs programming problems are now resolved

Known Issues:

- Small amount of analog audio buzz has been detected when no sound is being transmitted

- USB ports don’t recover when fiber / copper is removed, you need to reboot the unit

- About 1 in every 10 upgrades fail, just repeat the steps

- Restarting a Mac mini with OSX10.14 the USB doesn’t always resume, reboot the TX

- The Mac mini with OSX 10.14 occasional fails to resume from sleep, reboot the TX