BlackBox-LES FAMILY RELEASES

------------------------------------------------------------------------

This document brings all new features and bug fixes regarding

LES2800A-1 - LES2800A-48 versions (Including BETA

versions which have a letter after the version number).

------------------------------------------------------------------------

V_1.3.9  Oct/15/03 :  (official release; upgrade from V_1.3.8.1)

a) New features

* None

b) Bug fixes

* rfc2217 commands were not being properly handled by socket when no

data buffering or sniffing was enabled

* CrossTalk in bad console cables would halt a normal boot

* Command "updatefiles" was failing when the files under

/tmp/upd/<pathname> had subdirectories

* When user logged into the ACS using SecureCRT, telnet, or SSH and

disconnected after the login prompt they would get the following error

* pam_authenticate : Error in service module Removed message due to

/etc/motd

* crontab -e was not working

* "Password" in the pm utility was not working correctly

* Alias command removed from pm utility

* ts_menu with different Escape char was not working

* ports do not release when connecting to LDAP server

* The serial speed for ports configured as PPP was not set properly

c) Change Log

* /etc/motd removed from LES

* "Existent feature (syslog) -> the configuration file

(/etc/syslog-ng.conf) was changed. The f_kernel and f_alerts in

/etc/syslog-ng/syslog-ng.conf were changed."

* Comments for parameter all.sttyCmd in /etc/portslave/pslave.conf

might let user think that it was valid only for TS profile because

of the expression "terminal port", and it was not true. Changed

the comments.

* /etc/pm.cyclades had the AlterPath PM prompt changed from "pm8>"

to "pm>". If that file was NOT included in /etc/config_files by

the user in an earlier FW version, the LES FW upgrade will force

the user to either upgrade the AlterPath PM boxes to the newest FW

or to edit the /etc/pm.cyclades file in the LES, change the prompt

back to "pm8>", to include that file in /etc/config_files, to

restart pmd process and to run saveconf to have the LES

communicating with the AlterPath PM again.

PM versions before 1.0.9 have the "prompt pm8>"

PM versions after that have prompt "pm>"

/etc/pm.cyclades defines the prompt as "pm8>" for all LES versions before

V_1.3.7. It defines as "pm>" for V_1.3.8 and later

------------------------------------------------------------------------

V_1.3.8.1  Sep/18/03 :  (official release; upgrade from V_1.3.8)

a) New features

* None

b) Bug fixes

* Applied latest patches until openssh 3.7.1

c) Change Log

# The patch is a fix to buffer.c in openssh. The patch updates 3.6.1p1

to the current code level (3.7.1).

------------------------------------------------------------------------

V_1.3.8  Aug/11/03 :  (official release; upgrade from V_1.3.7)

a) New features

* SSL V2 reenabled in the FW (it was disabled in previous FW

version) and made configurable. This way, IE can work with SSL3 or

SSL2 whereas the existent buggy Netscape and Mozila can work with

SSL2.

* The upgrade process from V_1.3.6 or older to V_1.3.7 required the

user to read the upgrade notes and to be present in the console to

proceed with the upgrade process (copying configuration files from

old sector to new sector in flash). Now this process is automatic.

FW versions 1.3.6 or older that get V_1.3.8 don't require users in

the console to copy configuration files from the old sector to the

new sector.

* Hardened the code through disabling services daytime and time and

rejecting timestamp

b) Bug fixes

* saveconf was very slow to save configuration files in flash and

was generating R/W temp files before saving them into flash

* RFC2217 not working when user did not configure data buffering or

sniffing for a given serial port

* WEB would cap in 40 characters the users field before copying the

field to the proper configuration file (pslave.conf). Now the

limit is 256 characters.

* CrossTalk in bad console cables made the unit not boot if the

console cable did not have console hooked up

* Billing SW: It was accepted any combination of <CR> and <LF> as

the end of record. The problem occurred when the application got

from TTY these two characters in two

* different reads... Socket CAS was handling telnet command NOP as data

* pam was generating a deceiving error saying "unable to set group

membership for user (err=-1)". It was a incompatibility between

sshd (set_creds) and pam_groups (set_creds). The

pam_groups issue a system call that requires root privilege but

sshd already change the privilege to the user just logged in.

Removed the module pam_groups from the sshd service (pam.conf).

c) Change Log

* SSL2 enabled or disabled through configuration file

(/etc/ssl_version.conf). The user will choose between SSLv2,

SSLv3, and SSLv23 (default).

------------------------------------------------------------------------

V_1.3.7  Jun/30/03 :  (official release; upgrade from V_1.3.6)

a) New features

* Upgrade of OpenSSH (from 3.5.p1 to 3.6.1p1)

* Upgrade of Openssl to 0.9.7b

* Allows Radius Server to specify the serial ports the user can access

* tstest with chat string support

* Dynamic serial port allocation (hunting group; pool of serial

ports) support

* Flash relayout allowing more room for upgrade

* Support to LDAPDownLocal authentication

* SSH-2 break extension support

* performance improvement (transfer rate over serial ports). This

feature affects the meaning of the parameters all.DTR_reset,

all.auto_answer_output, and all.auto_answer_input

* support to change and control (FW and Configuration) to work with

new Cyclades product

b) Bug fixes

* Radius and callback was not working properly (no call back)

* "W" command showing a wrong pid  for ports TS profile

* Radius was sending a wrong NAS-Port-Id to Radius Server

* Changed DHCP client to keep trying to get an IP address forever if

configured as "1"

* If a user belongs to more than one group he can not access LES

serial port properly

c) Change Log

* Hunting group: added some new parameters in

/etc/portslave/pslave.conf (all.pool_ipno, all.pool_serverfarm,

all.pool_socket_port)

* Flash relayout: The customer must execute a shell script

(restore_from_old_script) as soon he boots the new version. This

is mentioned in the upgrade notes.

* added /webs/web/appl/close.gif, /webs/web/appl/refresh.gif,

/webs/web/appl/colorSet.conf

- java applet now pops up when you connect. There is a refresh and

close icon that users can click on. The refresh button is used to

reconnect to the server. The close icon is used to close the

* window of the popup. LdapDownLocal:

. change in /etc/portslave/pslave.conf

. changed WEB interface to support new value to authentication

type parameter

. changed snmpd to support new value to authentication type

parameter

*     . change din /etc/pam.conf (added new service ldapdownlocal)

SSH-2 break extension: (support to "Session Channel Break

Extension - draft-ietf-secsh-break-00.txt")

. implemented client and server.

. break interval -> change in /etc/portslave/pslave.conf

* (added parameter all.break_interval) performance improvement

. change in /etc/portslave/pslave.conf (included new value to

* all.sniff_mode) support to change and control (FW and Configuration)

. saveconf and restoreconf -> have more options

* . adduser -> allow to add user with root privileges existent

feature (DHCP client) -> the following files were changed :

/bin/handle_dhcp (now this script shell does the ifconfig commands

to set the IP address to eth0) and /etc/network/dhcpcd_cmd.

* existent feaute (default route) -> the /etc/network/st_routes was

changed. The option "metric 3" was inserted in the definition of

the default route.

------------------------------------------------------------------------

V_1.3.6  Mar/21/03 :  (official release; upgrade from V_1.3.5)

a) New features

* Power Management. Allows users connect IPDUs (Inteligent Power

Distribution Unit) from Cyclades and some other vendors (Baytech

and Sentry) to Cyclades' Console Servers and manage the outlets

used to power the Servers.

* Upgrade of OpenSSL (from 0.9.6g to 0.9.7)

* Upgrade of ucd-snmp (from 4.2.4.pre1 to 4.2.6)

* Upgrade of Busybox to 0.60.5. (include support to "top" command).

* Upgrade of DHCP to 1.3.22

* IP buffering for Billing applications (LES100 only)

b) Bug fixes

* LES100: Modbus daemon failed to start (modbus log was showing

"serial buffer size incompatible")

* Telnet/SSH connections with Data Buffering are locked after NFS

server goes down

* Protocol socket_server ignores the [more] data buffer menu command

over telnet

* Wizard for DB is setting the parameter *.data_buffering wrongly

when the value has more than 5 digits

* PPP connection from a Windows 2000 would not be established unless

cb_script line was commented out in pslave.conf

c) Change Log

* New feature (Power Management):

New directory/files:

- pmd/

- pmd/*

- cyclades/etc/init.d/pmd

Files changed:

- cyclades/etc/inittab

New parameters were added in webs configuration, serial ports

section : Protocol (the ipdu protocol was included), IPDU type, PM

users, PM number of outlets, PM outlets and PM hotkey.

- The same parameters above were added in the Cyclades MIBs.

- The process pmd was included in the webs administration to

restart processes.

- The Link Administration > Power Management was created to manage

the IPDU's outlets.

* The following commands were removed: radinit, last, ssh-keygen and

sftp. The reason was that the code grew big and the zImage was not

fitting anymore in flash.

* New Feature (IP buffering)

variables billing_records and billing_timeout added in

/etc/portslave/pslave.conf.

Comments: RAMDISK inodes from 1024 to 1152 (128 more).

*   Existent feature (Dial-in authentication by CHAP protocol) ->

created the directory "/etc/ppp" and the files chap-secret and

options.ttyXX.

* A new command line option was added to the tstest program: the "-I

<initchat>".

So, the command to do port conversation without navigating in the

menu should be:

tstest -l <#port> -s <baudrate> -I <initchat_string>

The command "tstest -?" will display all options available:

-l #port      - Serial port number [1 to 32]

-s speed      - Baud rate

-p parity     - Parity even, odd, none

-f flow       - Flow control hard, soft, none

-d DataLength - Number of bits from 5 to 8

-b            - Send break 0.25 to 0.5 seconds long

-B interval   - Send break [1-5] seconds long

-T interval   - Toggle DTR [1-5] seconds long

-t            - Toggle DTR forever

-R interval   - Toggle RTS [1-5] seconds long

-r            - Toggle RTS forever

-i            - Port conversation

-I <initchat> - Port conversation

-c            - doesn't change tty configuration/signals on open

-C            - doesn't restore tty configuration/signals on close

* Java applet has changed. Now to ssh to the port chosen, users can

just type the username and his/her password rather than typing

username:portnumber and then his/her password

* Existent feature (Dial-In profile) -> change in

/etc/portslave/pslave.conf (removed the callback from the default

of the pppoptions parameter).

*   Existent feature (CHAP authentication in Dial-In profile) -> add

two files /etc/ppp/chap-secrets and /etc/ppp/options.ttyXX.

*   Existent feature (busybox) -> upgrade version 0.60.2 to version

0.60.5 (included support to the "top" command  and the "ps" shows

new columns). The /etc/inittab file was changed because the order

to start the process was changed in the new busybox.

*   Existent feature (/bin/build_DB_ramdisk) -> change to not show

the messages from /etc/mke2fs and /etc/mount.

*   Existent feature (cyclades MIB) -> change to support new

PortSlave parameters and fixed some problems with object definitions.

*   Included Note about CHAP authentication (Chapter 3, section

Authentication)

------------------------------------------------------------------------

V_1.3.5  Jan/10/03 :  (official release; upgrade from V_1.3.4)

a) New features

* Upgrade of the WEB server (goahead v2.1.4)

* The WEB logic for access limit has changed. There will be 4

priority levels: user, monitor, administrator and full (root).

Each page will have a priority level associated with it; if the

page has monitor priority, all the users with privilege monitor,

administrator or full will have access to the page. The default

user groups will be root (full), admin (administrator), monitor

(monitor) and user (user). Also, the link list will be grouped

according to the user privilege. The common user, for now, will be

able to logout and to connect to serial ports, nothing more. In

order to make it effective, it's necessary to change the file

/etc/websum.conf with the one in the new zImage.

* Run Configuration implemented in WEB. A link was created in the

Administration section and, in the page, the administrator can

reload the portslave, the snmp and the syslog-ng configuration.

The signal_ras script was changed to fit this feature.

* Added a link called SNMP in the Configuration section. This

configuration is done in the same way as syslog-ng; by editing the

file.

* Changed the syslog-ng.conf file. The new configuration allows

syslog-ng to receive syslog messages from the Kernel.

* Implemented a new PortSlave parameter "all.telnet_client_mode".

This parameters allows the user to choose text or binary mode for

automatic telnet client.

* Implemented a new PorstSlave parameter "all.lf_suppress" to allow

some Windows telnet client to access Unix servers and not receive

double prompt.

* Implemented two new PortSlave parameters "all.auto_answer_input"

and "all.auto_answer_output" to allow PowerEdge Servers to display

BIOS' output when there's no connection (ssh or telnet) to that

serial port (given data buffering is active).

* Enhanced sniffer feature by allowing presenting or not the sniffer

menu

b) Bug fixes

* A problem in syslog data buffering was fixed. That would appear

when the parameter data_buffering is not enabled and the parameter

timestamp is enabled.

* When changing serial port configuration parameter like "sttyCmd"

and issuing the "signal_ras hup" command the serial port parameter

is not being reconfigured.

* If slave entries for all 48 ports of a LES3000 are added to the

pslave.conf file in the master the following message appears when

the slave is selected on the ts_menu first screen.

"Caution: You have exceeded the number of slaves allowed.

You may be invading your system's memory therefore affecting

the performance of this application..."

*   When doing ftp to upload /proc/flash/script file the 'put'

command does not work. Even changing the rights to the script file

(chmod 777) had no success.

* LES - SNMP

Fixed problem with to save configuration and to restart PortSlave

* by SNMP set. New Cyclades Logo replacing the old one

c) Change Log

* new feature (Access Limit by priority) -> change in

/etc/websum.conf (reconfigured user groups and access limits

according to the priority and added some more access list entried)

* new feature (Common Users access only application pages) -> files

web/read/{*.jar, *.conf, sportConnect.asp, connectPorts.asp} moved

to web/appl.

* new feature (complete Run Configuration) -> Link "Run

Configuration" inserted in the Administration section, inthe Web

Server Menu

* new feature (SNMP configuration) -> Link "SNMP" inserted in the

Configuration section, in the Web Server Menu

* new feature (Define the text/binary mode in automatic telnet

client) -> change in /etc/portslave/pslave.conf (added the

parameter all.telnet_client_mode)

* new feature (LF suppression) -> change in

/etc/portslave/pslave.conf (allows suppressing the last LF from

the CRLF sent by a Windows telnet client to avoid having double

prompt on screen when user accesses a Unix server through the CAS'

serial port)

* new feature (Probing mechanism) -> change in

/etc/portslave/pslave.conf (if a server probes the serial port by

sending a string the CAS answer with other string so BIOS can

start displaying. Input and output strings are configurable)

* existent feature (session sniffing) -> change in

/etc/portslave/pslave.conf (all.multiple_sessions can be

configured to present or not the sniffer menu)

* existent feature (syslog-ng receives syslog message from kernel)

-> change the syslog-ng.conf file (see the upgrade notes), change

in upgrade_110 file

* the command "w" is changed. The original version was renamed to

"w_ori". "w_cas" is a new command and it shows the information

about CAS sessions. The command "w" calls w_ori and w_cas.

* LES MIB for SNMP management

* Included new PortSlave Parameters in the LES MIB. The configuration

of the snmpd (/etc/snmp/snmpd.conf file) was changed. The upgrade

has to be done in two steps:

. First step.

.. Save the file /etc/snmp/snmpd.conf, if it was changed.

.. Edit the file /etc/config_files and remove the line related to

snmp.

.. Execute the command "saveconf" and reboot the LES.

. Second step.

.. if the file /etc/snmp/snmpd.conf was changed the user should

make his own

changes again.

------------------------------------------------------------------------

V_1.3.4  Nov/01/02 :  (official release; replaces V_1.3.3 for

LES100/400/800 HW with 32M RAM and LES1000/2000/3000 that supports at

least 2M RAM per serial port)

NOTE: Versions 1.2.X and on will only run on HW that supports 2M RAM per

serial port. Versions 1.1.X will run on HW units that support 1M and 2M

RAM per serial port. Version 1.3.4 and later will only run on

LES100/400/800 with 32M RAM and LES1000/2000/3000 with 32M/64M/128M RAM

respectively.

a) New features

* upgrade the OpenSSL version - new  0.9.6g

* LDAP support (Tacacs+, Radius and local authentication)

* Extended configuration through wizard

* Local linear data buffering

b) Bug fixes

* Files in flash can be protected by just setting the right permission

* Some bugs detected using WEB

* With local authentication turned on, reverse telnet to port, get

double echo.  Menu is garbaged.

* Reboot unit through WebUI causes uneccesary Web Server info to be

spit out.

* If no password is specified in Add System User page, then the

functionality gets all messed up.

* No Header/Title displayed on "Add System Group" page.

* LEDs are all on even without connection and data buffering. w

command presented wrong "from" information when connecting to a

serial port (telnet/ssh)

* set date/time through WEB was not working on months with 31 days

* It was not possible to make more than 16 simultaneous ssh sessions

to serial ports

* IdleTimeout ignored input data from the serial port to reset the

counter

* Konqueror 3.0.0 crashes web interface

* If one try to sniff and then assume a socket_server session, the

sniffed session always die (this is correct), but the sniffing

session doesn't assume the session.

------------------------------------------------------------------------

V_1.3.3  Aug/15/02 :  (official release; replaces V_1.3.2 for HW that

supports 2M RAM per serial port)

NOTE: Versions 1.2.X and on will only run on HW that supports 2M RAM per

serial port. Versions 1.1.X will run on HW units that support 1M and 2M

RAM per serial port.

a) New features

* upgrade the OpenSSH version - new  3.4p1

* PAM support (Tacacs+, Radius and local authentication)

* Alarm generation (use of syslog-ng 1.5.17)

* Time stamp in data buffering configurable

* Multiple sniffing configurable per port

* Basic configuration through wizard

* Custom programing of stty

* ModBus support (LES100)

* Multiple syslog servers support

* Socket client cabable of working with raw data (configurable)

* Data buffering continues even when the connection with the serial

port is established

b) Bug fixes

* Serial port configured as socket_ssh and with no authentication

was not working

* Dial-in presented problems when authentication was set to local

* When some SUN servers would reboot the LES would also reboot (if

watchdog timer was active) due to noise in the DTR line coming

from the SUN server. No problem found if the WDT was inactive.

* WEB in the LES had problems when configuring ipchains

* Time zone was not OK for some daemons (syslog, cron, and so forth)

c) Programs removed from Ramdisk due lack of memory (LES100, LES400, and

LES800 only).

arp, crontab, ex_keygen, fwset, ipchains, ipchains-restore, ipchains-save,

last, mke2fs, netstat, nohup, pftp, sftp, sftp-server, snmpd, snmptrap,

sysctl, tftpd, and wall

d) The ts_menu application now can also handle slave ports (clustering)

but the remote tcp port number in the slave ports must be always

configured (s<nn>.tty slave_ip:tcp_port_number). Before the tcp port

number parameter was optional.

------------------------------------------------------------------------

V_1.3.2  May/06/02 :  (official release; replaces V_1.3.1 for HW that

supports 2M RAM per serial port)

NOTE: Versions 1.2.X and on will only run on HW that supports 2M RAM per

serial port. Versions 1.1.X will run on HW units that support 1M and 2M

RAM per serial port.

a) New features

* upgrade the OpenSSH version - new  3.1p1

* upgrade the OpenSSL version - new 0.9.6c

* Zlib Version 1.1.4

* upgrade the SNMP version - new ucd-snmp-4.2.4pre1

* upgrade the Busybox version - new  0.60.2

* included support to binary mode in Telnet Client

* upgrade the PPPD version - new 2.4.1.

* add new parameters in saveconf and restoreconf commands.

* adduser : new parameter "-g <group_name or group_id>"  used to set

the user's initial login group. The group must exist.

* changed the parameter <comment> :  old "-g <comment>" and now "-c

<comment>"

* included new parameters in pslave.conf (all.tx_interval,

all.idletimeout, all.DontShowDBmenu, conf.group, conf.include,

conf.host_config)

* Fast Ethernet ? Maximun events handling per interrupt

WEB UI : new parameters from pslave.conf,  DHCP client Information

* - display the "/etc/dhcpc/dhcpcd-eth0.info" file HTTPS used by

default instead of HTTP for security reasons

b) Bug fixes

* Control-C was not working in single mode

* LES100: wrong initialization in RTS signal was locking the console

* Syslog with wrong timestamp

* If all ports were configured as telnet the LES would reboot if

socket_port was invalid

* poll_interval zero was affecting the transmission to the socket client

* Client telnet (CS8 raw mode)

* all.user ignored when protocol is socket_ssh and autentication local

* Sniff session: could no't assume session if there was data traffic

* Reboot with multiple ssh + sniff session + whatdog timer

* The mv -f differs in functionality from typical Linux OS's

* Wrong start time for active SSH sessions

* Ports were locking up in certain conditions

* SSH-2 was not working properly in certain conditions

* DTR LEDs would stay ON even without process using the port

* "cat" command, when the source and destination files is the same,

caused a recursive write, now results in an error saying the input

file is the output file.

* WEB UI : file edition bug fixes, crashes when working with Ip

Chains, bootconf parameters only valid after reboot,.

Authentication type did not read/write properly, change password

screen did not alter one to an invalid user, serial ports only

shown up to 30 ports (LES2000), change password screen displayed

edit boxes for passwords at bottom of screen, add new users -

there was not way to assign a user to a different group than the

default, and problem with "Load/Save Configuration" when using FTP

server.

* Data buffering method switching from local buffering to NFS files

or vice versa would halt buffering

* Removal of data buffering file would not force the firmware to

recreate the data file for further buffering

------------------------------------------------------------------------

V_1.3.1  Feb/18/02 :  (official release; replaces V_1.3.0 for HW that

supports 2M RAM per serial port)

NOTE: Versions 1.2.X and on will only run on HW that supports 2M RAM per

serial port. Versions 1.1.X will run on HW units that support 1M and 2M

RAM per serial port.

a) New features

* LES100 support

* New protocol: raw_mode for transparent transfer between serial

device and application

b) Bug fixes

* Sniff session: when there was no user, admin_user configured and

authentication was set to none anyone could sniff the session on a

given port

* User's access: all.users ignored when all.protocol was set to

socket_ssh and all.authtype was set to local

* WMI: file edition inserting ^M at the end of each line, not

handling properly % and \

* WMI: Editing a non-existant file causes a crash.

* WMI:  The status messages after editing a file were reversed

(success was claimed on failure, and vice-versa).

* Serial driver: flow control done by software changed to done by

the controller

* Serial driver: character 0xff was filtered out instead of

forwarded to the serial device

* WMI: if hostname e hosts are  different the web server aborted

with no message

* Sniff session and ssh: if the main session closed before the admin

could choose one option, when establishing the second session, the

sshd would abort.

* User's access: when using RSA/DSA the user field in pslave.conf

was not considered

* When booting the LES in single mode, the 10BT was running with less

performance

------------------------------------------------------------------------

V_1.3.0  Oct/15/01 :  (official release; replaces V_1.2.0 for HW that

supports 2M RAM per serial port)

NOTE: Versions 1.2.X and on will only run on HW that supports 2M RAM per

serial port. Versions 1.1.X will run on HW units that support 1M and 2M

RAM per serial port.

a) New features

* Clustering (Daisy Chain) support

* Session Sniffing support

* WEB management/configuration (beta release)

b) Bug fixes

* None

------------------------------------------------------------------------

V_1.2.0  Sep/17/01 :  (official release; replaces V_1.1.2 for HW that

supports 2M RAM per serial port)

NOTE: Versions 1.2.X and on will only run on HW that supports 2M RAM per

serial port. Versions 1.1.X will run on HW units that support 1M and 2M

RAM per serial port.

a) New features

* IP filtering support

* DHCP support

* Application that provides menu oriented screen for those who do

not want to handle command line

b) Bug fixes

* Static route learned from Radius Server was not inserted in the

routing table if the interface got the IP address from the Radius

Server and it was different from the one configured in pslave.conf

* Bootconf application with problems with back space and reading an

invalid configuration

* Data buffering stopped working if a port got more than one SSH

session at the same time

------------------------------------------------------------------------

V_1.1.2  Aug/10/01 :  (official release; replaces V_1.1.1)

a) New features

* Data buffering allows non cycle network file

* User auto created to allow automatic telnet/login as soon as user

is logged into the serial port (Terminal Server profile)

* Disable/Enable presentation of data buffering menu when user

connects to the serial port

* Created restriction access per port (e.g. port #1 can allow users

X, Y and Z, whereas port #2 allows only user L, and so forth)

* New authentication type created to allow trying access to Radius

Server first and, if there's NO answer, tries in the local

database. If the Server refuses access, no attempt is made in the

local database.

* Root directory changed from / to /root (see upgrade_112 file for

details)

* Telnet client using configurable TCP port number

b) Bug fixes

* Attempts to connect from SSH client version 1.2 or later used SSH2

and was not working. Only disabling on the SSH client the SSH2

attempt would allow a connection to the serial port or LES.

------------------------------------------------------------------------

V_1.1.1  Jul/16/01 :  (official release; replaces V_1.1.0)

a) New features

* Data buffering allows network file

* Ethernet mode configurable

b) Bug fixes

* Put lockdir in SSH

* Ethernet 10BT sometimes presented performance problem in inbound

traffic

* Console autosense capability led sometimes to wrong settings (like

7E1) when hooking up the console cable

------------------------------------------------------------------------

V_1.1.0  Jun/11/01 :  (official release; replaces V_1.0.1)

a) New features

* Data Buffering per port

* Authentication can be done using one source (Radius or local) or

two (Radius and then local, or local and then Radius)

* NTP client support

* SSH2 support

* SSH allows use of IP to address directly the physical port

* SSH can send break through configurable strings present in pslave.conf

b) Bug fixes

* Error when configuring the /etc/hosts and /etc/hostname. portslave

would not start eth0 if the hosts had the same name.

* Simultaneous SSH sessions on all ports could cause Panic due to

lack of RAM.

* Blanks after a string in pslave.conf caused problems in the

interpretation

* SSH sessions did not allow serial ports with configurations other

than 8N1

* SSH sessions had some problems when with CR/LF conversion

* Telnet to remote hosts was not passing the correct terminal type

------------------------------------------------------------------------

V_1.0.1  Mar/13/01 :  (official release; replaces V_1.0.0)

a) New features

* None

b) Bug fixes

* SSH to the box was not allowed, just to the physical port

* Portslave was overwriting ifcfg_eth0 when it was not necessary

------------------------------------------------------------------------

V_1.0.0  Mar/12/01 :  (official release; first release)

a) New features

* TCP/IP RAS, TS and Console Manager profiles in an Embedded Linux

* SSH, telnet, ftp, PPP and SLIP, 10/100BT

* Spurious Break Eliminator

b) Bug fixes

* None

------------------------------------------------------------------------